# Design and Implementation

## 1. Introduction

A firewall is a network security device that monitors and controls incoming and outgoing network traffic according to predefined security rules. In this project, a firewall is placed between the Internet and the university web server to protect the server from unauthorized access and malicious traffic.

The firewall acts as a security barrier between the untrusted Internet and the trusted university server network. Only required services such as HTTP and HTTPS are permitted, while unauthorized traffic is blocked.

## 2. Proposed Network Architecture

The proposed network consists of the following components:

Internet → Firewall/Router → Web Server

The Internet represents the untrusted external network. The firewall is responsible for inspecting network packets and applying access-control rules. The web server is placed behind the firewall in the protected network.

### Architecture

INTERNET

|

|

+---------------+

|    FIREWALL   |

| Router + ACL  |

+---------------+

|

|

+---------------+

|   WEB SERVER  |

| 192.168.10.10 |

+---------------+

## 3. Network Components

| Component | Purpose |
| --- | --- |
| Internet | Represents external/untrusted users |
| Firewall/Router | Filters incoming and outgoing traffic |
| Web Server | Hosts the university web application |
| Access Control List (ACL) | Defines permitted and denied traffic |
| Cisco Packet Tracer | Used to simulate and test the network |

## 4. IP Addressing Scheme

| Device | Interface | IP Address | Purpose |
| --- | --- | --- | --- |
| Firewall/Router | Outside | 203.0.113.1/24 | Internet-facing interface |
| Firewall/Router | Inside | 192.168.10.1/24 | Web-server network gateway |
| Web Server | NIC | 192.168.10.10/24 | University web server |
| External PC | NIC | 203.0.113.10/24 | Simulated Internet user |

The firewall uses two network interfaces:

- Outside interface: Connected to the Internet.

- Inside interface: Connected to the protected web server.

## 5. Firewall Policy

The firewall follows the principle of allowing only necessary traffic and denying unauthorized traffic.

The university web server needs to provide web services to legitimate Internet users. Therefore, HTTP and HTTPS traffic are permitted.

### Firewall Rules

| Rule No. | Source | Destination | Protocol/Port | Action | Purpose |
| --- | --- | --- | --- | --- | --- |
| 1 | Internet | Web Server | HTTP (TCP 80) | Allow | Allow web access |
| 2 | Internet | Web Server | HTTPS (TCP 443) | Allow | Allow secure web access |
| 3 | Internet | Web Server | SSH (TCP 22) | Deny | Prevent external administration |
| 4 | Internet | Web Server | Telnet (TCP 23) | Deny | Prevent insecure remote access |
| 5 | Internet | Web Server | ICMP | Deny/Restrict | Prevent unnecessary ping access |
| 6 | Any | Any | Other traffic | Deny | Block unauthorized services |
| 7 | Web Server | Internet | Required traffic | Allow | Permit necessary outgoing communication |

## 6. Implementation Using Cisco Packet Tracer

The firewall functionality is simulated using a Cisco router with Access Control Lists (ACLs).

### Step 1: Create the topology

The following devices are placed in Cisco Packet Tracer:

- 1 Router

- 1 Switch

- 1 Web Server

- 1 External PC

The devices are connected using Ethernet cables.

External PC

|

|

Internet Network

|

[Router]

Firewall ACL

|

[Switch]

|

[Web Server]

### Step 2: Configure the router interfaces

The router's external interface is connected to the Internet-side network, while the internal interface is connected to the web-server network.

Example:

Outside Interface: 203.0.113.1/24

Inside Interface:  192.168.10.1/24

### Step 3: Configure the web server

The web server is assigned:

IP Address:      192.168.10.10

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.10.1

HTTP/HTTPS services are enabled on the server.

### Step 4: Configure the firewall ACL

An extended ACL is configured on the router to control traffic reaching the web server.

Example ACL policy:

Allow TCP port 80

Allow TCP port 443

Deny TCP port 22

Deny TCP port 23

Deny unauthorized traffic

The ACL is applied to the Internet-facing interface in the appropriate direction.

## 7. Access-Control Logic

The firewall processes packets according to the configured rules.

Incoming Packet

|

v

Firewall ACL

|

+---- HTTP/HTTPS ----> ALLOW ----> Web Server

|

+---- SSH ------------> DENY

|

+---- Telnet ---------> DENY

|

+---- Other Traffic --> DENY

This ensures that the web server is accessible to legitimate web users while reducing exposure to unauthorized services.

## 8. Outgoing Traffic Control

The firewall also controls traffic originating from the web server.

Only required outbound communication is permitted. Unnecessary or unauthorized outbound traffic is restricted.

This prevents the server from being used to communicate with unauthorized external services.

## 9. Security Testing

After implementing the firewall policy, connectivity tests are performed to verify whether the rules work correctly.

### Test 1: HTTP Connectivity

An external PC attempts to access the web server using HTTP.

Expected Result: Connection successful.

External PC → TCP Port 80 → Firewall → Web Server

ALLOW

### Test 2: HTTPS Connectivity

An external PC attempts to access the web server using HTTPS.

Expected Result: Connection successful.

External PC → TCP Port 443 → Firewall → Web Server

ALLOW

### Test 3: SSH Access

An external PC attempts to connect to the web server using SSH.

Expected Result: Connection blocked.

External PC → TCP Port 22 → Firewall

DENY

### Test 4: Telnet Access

An external PC attempts to access the server using Telnet.

Expected Result: Connection blocked.

External PC → TCP Port 23 → Firewall

DENY

### Test 5: Unauthorized Traffic

An external PC generates traffic using an unauthorized service/port.

Expected Result: Traffic is blocked by the firewall ACL.

## 10. Simulation Mode Testing

Cisco Packet Tracer's Simulation Mode is used to observe how packets travel through the network.

The testing demonstrates:

- Packet generated by the external PC.

- Packet reaches the firewall/router.

- Firewall checks the ACL.

- If the traffic matches an ALLOW rule, it is forwarded to the web server.

- If the traffic matches a DENY rule, the packet is discarded.

This provides visual evidence that the firewall policy is functioning correctly.

## 11. Expected Results

| Test | Expected Result |
| --- | --- |
| HTTP access | Successful |
| HTTPS access | Successful |
| SSH access | Blocked |
| Telnet access | Blocked |
| Unauthorized service | Blocked |
| Required outbound traffic | Allowed |

The testing demonstrates that the firewall successfully controls traffic according to the defined security policy.

## 12. Security Benefits

The proposed firewall implementation provides the following security benefits:

- Prevents unauthorized access to the web server.

- Allows only required web services.

- Blocks insecure services such as Telnet.

- Restricts external administrative access.

- Reduces the attack surface of the server.

- Provides controlled communication between the Internet and the protected network.

- Supports centralized access-control policies.

- Allows security testing through Cisco Packet Tracer simulation.

## 13. Conclusion

The secure firewall policy was designed and implemented to protect the university web server from unauthorized Internet traffic. The firewall uses ACL-based access-control rules to permit legitimate HTTP and HTTPS traffic while blocking unnecessary and potentially dangerous services.

The Cisco Packet Tracer simulation demonstrates that authorized traffic can reach the web server successfully, whereas unauthorized traffic is rejected by the firewall. Therefore, the implemented firewall policy provides an effective security boundary between the Internet and the university web server.